ProUSB Card Lock Management Software [Guidance version]

Manual
1. Overview
Features:
● proUSB refers to Professional USB. With advanced driverlessness USB technology, without installing Driver File, It can be easily and conveniently used when there is Windows System in the PC with USB function.
● Easy lock configuration. Setup for Room No. and Clock isn’t needed to be operated. Only Read System Card (Authorized Card) three times over the Lock inductive area and use the corresponding Guest Card to open the door, Room No. and Clock is going to have accessed into Door Lock automatically.
● It is radical reform for proUSB System that having Lock Time kept pace with computer time theoretically in order to eliminate cumulative error (general speaking, the Time Difference between the lock and computer will be kept within 5 minutes). 
●Individuated setup for the functions such as Alarm, Close Door Prompt, Do-Not-Disturb When Inside Lock etc.
● Software interface use brand new Windows-7 Style, and almost its every Setup step accompanies with indication. Any kinds of report forms can be exported as Excel files which is convenient for users’ sorts of typeset requirement.
● Abundant Room Status is more obviously embodied in the aspects of Reservation and Hour-Rate Room as well as Group Reservation etc.
● Abundant System Indication function. Move the Mouse to a certain position and still for a while, the system will automatically display the Operational Manual or Function Indication, liked[image: image1.png]Press Fi For Heb|



, above that position.
● One side of the Card Reader is connected by the custom-made wire with button to ensure the right connection. There is “Card-laying Place” on Card Reader for laying the card that is very obvious to find.
2. Installation
2.1. Preparation: 
PC installed Windows Operational System (XP, VISTA, WIN7, or Win2000) alone with at less one USB Post, which Solution is at least 1024 x 768.
2.2. Card Reader Installation:
Connect the Card Reader wire with computer’s USB port, when Green LED lights after two beeps, installation succeed.
[image: image2.png]


      [image: image3.png]Inductive Area
| CardHere

-

Handle

| Machnical
Key





Card Reader Appearance        Lock Exterior Appearance
2.3. Door Lock Software Installation:
The Door Lock Software is “Setup.Exe” (It might be copied into disc, or, kept in U-Disk or downloaded from Internet). 
Double click this software to installation. As normal, installation can be finished following the indication “Next” showing as below.
[image: image4.png]InstalShield Wizard

Preparing to Install

proUSBHotelCardSystem Setup s preparing the InstalShield Wizard
which wil guide you tiough the program setup process. Please wai.

Checking Dperating System Version




  [image: image5.png]1 prousBHotelCardSystem - InstallShield Wi

Welcome to the Instalishield Wizard for
prousBHotelCardSystem

The Instalshield(R) Wizard wilnstal proLSEHotelCardsystem
on your computer. To continue, clck Next

WARNING: This program i protected by copyright law and
internatianal treaties.





[image: image6.png]& proUsBHotelCardSystem - InstallShield Wizard

Destination Folder

Clck Nt to nstallto this Folder, or click Changs to nstal to 3 diferent folder

ﬁ Install proUsBHotelCardsyster to;
Ciprogram Files)

nge.

Instalhiel]

<ok ==





When this icon[image: image7.png]i

prosBHotelC
" ardSystem



appears on desktop,Congratulations,software installs correctly.
2.4. Door Lock Software application
Double click [image: image8.png]i

prosBHotelC
" ardSystem



 to startup the software. Then, choose defaulted operator “Admin” and enter without password in the Logon Interface. 
[image: image9.png]Welcome to use Card Lock

- Cmtm'_ %

__!.——passwnm

/0K





The main window will appear as follows:
[image: image10.png]ersioni BCLMS-Y9.25.

Hotel Lodenmemaeisy

& Rece

gr Daor Lock Configuration

\? System Parameter Management

X Exitx] [ ©U8SBe: cardLock





There are six functional parts of this system. Information included Edition Details, Account User, and Name of Hotel shows simultaneously.
3. System maintenance (System Setup)
Click[image: image11.png]l a System Parameter Management



in the main window to gain access into System Maintenance Interface which has three options[image: image12.png]P,

[options| System Registe | Operator Management



.
3.1. System general option:
Hotel Name, Default Check-out Time, Room Status After Check-out can be input and the function that Guest Card which can open the Inside Locks can be controlled in this interface. 
Click Button Save after operation
[image: image13.png]Options |

Hotel Name

Path of DB Backup Check-Out

VIP Check-Out
Hotel International

D:\proUSB_DBBak 12:00 H 1400

% Check-out to vacant status, Check-outto cleaning status
] Guest card can open the door which deadbolt s locked

 Save




System Extending Option: 
Select the System Option and then press F5, select the functions based on user’s requirement. Then save and issue one System Card to swipe over the Lock Conductive area to set up this option into door lock.
[image: image14.png]Extra Options

Alarm while Lockpin forced
Remind to close door
Do not Disturb

] Lock After Release Knob

I Music

Ignore Overwrite Function

Return To Default




Note: Three lock pins should be known in advance: Slope Lock pin, Thieving-prevented Lock pin, and Deadbolt.

[image: image15.png]Lockpin
Lach

DeadBolt ——




< Lock Inner Profile>
Alarm Option:

For Lockpin Latch are maliciously operated or the door is not properly closed: Normally, the Lockpin Latch should enter into the key hole after door closed. If Lockpin Latch is blocked after door closes, Door Lock would alarm.
Prompt For Closing Door Option: 
Within 6 seconds after opening the door by Guest Card, the door still open, Door Lock will sound three beeps for reminding Guest of closing door. Only have Guest Card had such function.
Inside Lock and No Disturbing Option: 
The indicated light will dimly flash every 3 seconds to indicate NO BOTHER when Deadbolt is locked. 

Release Handle Option: 
After swiping card, Mini Motor will turn clockwise, press down the handle right now to open the door. When releasing the handle, Mini Motor will turn counter-clockwise. 
If omit this option, the Mini Motor will turn counter-clockwise at once and the door can’t be opened again when press down the handle.
Music Indication Option: 
After swiping card, there is one sound of music to indicate user to open the door. If omit this option, the sound will be “Beep” only.
Cover Function Option: 
The function of this Option is that the information of all cards, except Guest Cards, are allowed covering the one of previous cards according to the type of card which means that only the same type card can do this operation.

If select this option, the Cover Function is unavailable; if don’t, when issuing new Staff Card, user can consider whether select[image: image16.png]Overwrite all the previous Master Cards



 to decide whether revoke the information of the previous card after opening door. 
3.2. System Registration:

Selecting “system Registration” Option under the System Maintenance Interface, there will be a menu as below:

[image: image17.png]System Registe

Steps Of Registe

Step 1: Get the Device No. and tell it to the vender

01FF EDOB

No. and Click Registe

Step 2: Input the Se

§ Registe

For Door Lock Installation Only

Note

Put a master card which can open door on the Reader when you Reinstall the Lock Management
System and Registe it




Note: 1). Machine Code: Unique Serial Number of Card Reader.
2). It should be registered over again by using the new Machine Code when changing Card Reader.

3). If the system is registered initially, please place a valid card (Master Card or System Card) on the Card Reader and then registe.
             4). If Project Contractor installs this Software for lock testing, please choose this item[image: image18.png]vI{For Door Lock installation Onis




3.3. Operator Administration:

Selecting “Operator Management” Option under the System Maintenance Interface, there will be a menu as follows:

[image: image19.png]Operator Management|

List Of Operators

Type of Operator
7% Add a New Operator

* Administrator

+1 Remove a Operator

Manager

General Operator 8 Modify Operator Information




Note: 1). “Super” refers to Super Administrator, even though it is cancelled, it will be automatically added after logging on system afresh.
2). Difference between “Super” and “Admin”: 
“Super” can modify the Popedom List of all operator.
3). Account for Operator can’t be same.
4). New added operator is defaulted without password. Operator can modify password by clicking[image: image20.png]


 at the right side in this interface after logging on and selecting his account.
4. Rooms Definition
(Room Number Editing)

Gain access into Room Definition Interface by clicking[image: image21.png]ing Management




 under the Main Window. 
Click the “Order Button” at the upper-left of this interface to easily operate to define room following the indication according to the hotel practical situation.
      [image: image22.png]Rooms Defination

%)

List Of Buildings
N Name 71 Insert a New Bui
| =t RemoveaBuilting |
| 8 Modity Builting Name |

Insert Room(s)

+% Remove a Room

) Modify Room Information
List Of Rooms---Holiday Inn s

FloorNo.  [RoomNo.  [Type Day Price | Hour Rate Start-up [ Hour Rate Price [Status Ca
i 101 Standard 123 30 20 Vacant 10





The function of Adding Room in Batch Bulk:
Step1. Click[image: image23.png]Insert Room(s)




Step2. Input Floor Number range separated by “-“according to practical situation. For instance, if 1 to 5 floor, input 1-5.

[image: image24.png]Insert Room(s)

Flease specily the Floor No betwesn 098 ot input
the Floortange such as 116

15





Step3. To signify that there are 18 rooms in each floor, just input the first room number in first floor and the last room number in last floor, separate them by “-“ , for instance 101-518.

 If some rooms don’t exist, add them together with exist ones firstly and then cancel.
[image: image25.png]Insert Room(s)

Please spesify the Guestioom No. ofreom range
such s 101-1620

101519





Step4. Input Room Type, Room Price, and Card Quantity etc. in turn following the indication to finish adding room number in batch bulk.
Special Note: For the practical situation, there may be not some rooms or some rooms’ details are not in accordant to each other, in this case, add them firstly in batch bulk when operating Definition and then cancel or modify.
5. Door Lock Setup
5.1. Authorization for Door Lock:

Click[image: image26.png]@F Daor Lock Configuration



 in the Main Interface to gain access into Door Lock Setup Interface. There are operational indications at the down-end of this interface. 
One Authorizing Card for one hotel.
[image: image27.png]P,

Normal Configuration System Card
4 system Card !
S Card Holder b

Extra Configuration

Memo
2 Room No. Setting Card

& Clock Setting Card

[G] Groupio. Seting Card [] Overwrite all the previous system cards

/ lssue Card
X Return to Main[x]

Guide
Usually, Use System-Card Only





5.2. Room Number Setup: 
Input the designated Room Number to the corresponding Door Lock (This function is to take good care of our old clients’ using habit).

     As usual, use an Authorizing Card to swipe over the Lock Sensor for three times and then open the door directly by Guest Card of this room, the room number will have been setup into Door Lock.
[image: image28.png]Room No. Setting Card

Card Holder d

Building-Room No.

2 Room No. Setting Card -
Q search
Set room o t door lock|

Overw
Hcards

/ lssue Card

all the previous room no. setting





5.3. Door Lock Clock Setup:

     Set up the appointed time into the corresponding Door Lock (This function is to take good care of our old clients’ using habit.).

     Generally, use an Authorizing Card to swipe over the Lock Sensor for three times and then open the door directly by Guest Card of this room, the Card-issuing Time for this Guest Card will have been setup into Door Lock.
[image: image29.png]Clock Setting Card

Card Holder .

Current Time.
2010-1-25 18:31:20

& Clock Setting Card

- Overwrite all the previous dateftime setting
cards

/ lssue Card





5.4. Group Number Setup: To group the Door Lock.
Pleas apply this function when you are in below conditions:
1). One card for many floors but not the whole floors of building (for this purpose, Building Card can be use).

2). One card for some rooms of half floor or one floor.

3). One card for opening any room controlled under this System.

Group Number Setup Option:
[image: image30.png]GroupNo. Setting Card

Clear One Group
Set One Group
Clear Al Groups
Set All Groups





Step1. For initializing Grouping Setup of Door Locks, please click[image: image31.png]GroupNo. Setting Card



in the Door Lock Setup Interface, select[image: image32.png]Clear All Groups



 for issuing a card named “Delete All Group Number” (DAGN Card), use this DAGN Card to delete group number by swiping it over these Lock Sensors. After hearing two beeps, deletion finishes. 
Step2. For grouping the number(0-255) to these Door Lock, please click[image: image33.png]GroupNo. Setting Card



in Door Lock Setup Interface and then select[image: image34.png]Set One Group



, input the needed group number, click ”Issue” to make one card named “One Group Number Setup”(OGNS Card). Use this OGNS Card to set up group number for these Door Locks by swiping it over these Lock Sensors. After hearing two beeps, setup finishes.
Note for Group Number:
1).There is 256 groups for one lock which means one lock can simultaneously set up several Group Numbers.
2).To canceling one certain Group Number of lock, firstly issue a card named “Delete One Group Number” (DOGN Card), use DOGN Card to swipe over these Lock Sensors. After hearing two beeps, deletion finishes.
3). To have one certain lock been subject to all Group Numbers, firstly issue one card named “Setup All Group Numbers” (SAGN Cards), use it to swipe over these Lock Sensors. After hearing two beeps, Setup finishes.
Step3. After finishing Group Number Setup, issue one “Group Controlling Card” for opening all locks of this group.
          How to issue: 
Click[image: image35.png]


in the Main Window and then click[image: image36.png]=& Group Card



, input information about User Name, Date of Validity, Period of Time, Group Number and the like, and click “Issue” for finishing.

For instance：Inputting Group Number “5” stands for that this card can open all the locks which are subject to Group Number 5.

[image: image37.png]Group Card

Holder ‘@

Date of Expiry Time
2011- 125 5| [00:00 = > [23:59 [0
Group No.

Overwrite all the previous Group Cards
Unlock Public Door
Unlock Deadbolt Locked

/ lssue Card




6. Card for hotel staff
Click[image: image38.png]


in the Main Window, there will be an interface as below:
[image: image39.png]Keys For Open Door Floor Card

=@ Floor Card &
Holder

=@ Master Card

ing Name Floor No.
= Emergency Card ][ o]
Date of Expiry Time
=6 Group Card —
2010- 2-24 o] [00:00 = > [23:58 [0
Functional Cards Memo
@ Record card
[7] Overwrite all the previous Floor Cards
® Lostcard N
(] Unlock Public Door
@ Terminate Card (] Unlock Deadbolt Locked

/ lssue Card
X Return to Main[x]





Clicking the buttons at the left side in this interface, the corresponding items will appear at the right side of this interface. The information can input into the blanks of these items according to the practical situation and then click ”Issue” to issue a Staff Card.
The use of Staff Card:
1). Floor Card: Use for opening all locks in a certain floor in a certain building.
2). Building Card: Use for opening all locks in a certain building.

3). Master Card: Use for opening all locks of Lock System.

4). Emergency Card: Use for opening all locks of Lock System and all the locks will remain unlock state. 
To disable the opening state, here are two solutions: 
A). Press down the handle and swipe the Emergency Card immediately; 
B). Use ordinary Cards (Master Card or Floor Card etc.) to open the door one time, the lock will be free from opening state.
5). Group Card: Use for opening the group locks. For details, please refer to “Group Number Setup” in “Door Lock Setup”.
6). Record Card: For MIFARE 1 Lock, its Record Card is S70 Card using for picking up the door-opening record; For TEMIC Lock, its Record Card is T5557 or T5567 Card using as Directing Card which swipe over the Lock Sensor before using Collector to pick up the door-opening record.
7). Loss-Report Card: If the card lose, please input or select Card Number of the lost card to issue the Loss-Report Card. Use it to swipe the Door Lock that the lost card can open.
8). Check-out Card: For server. 
Normally, there are two cards taken by servers. One is Floor Card using for opening door. Another is Check-out Card, which is used to disable the unlock function of the previous Guest Card without affecting the one of new Guest Card.
7. Reports
7.1. Check a variety of records:
Click[image: image40.png]7 Report



on the Main Window, there will be showed as follows:
[image: image41.png]Report of Cards





Choose Want-to-be-checked Record by clicking the upper-middle sheet which includes Card Record, Client Record, and Door-opening Record. Select[image: image42.png][ Option



 for Record Scope at the upper-right side.
[image: image43.png][ Option

Check In Check Out
2010- 1118 o] > [2010- 125 B 2010- 1118 | -> 20991230 B
Type Room No.

Operator Code

Order

C Search

v/ 0K
X Cancel





Note:
 (1). All records can be exported as Excel File. The file can be typed or saved or printed and the like.
(2). Under the condition of default, only had the Cancelled Card and Expired Card and Checked-out Card deleted when clicking on the button “Cancel”. Supposed other cards want to be deleted, except the above mentioned ones, please move the mouse to the appointed record and press the button “Del” on keyboard. The icon[image: image44.png]


will be appeared at the left side of the record, and then click [image: image45.png]


 to delete.
(3). It is the system data for factory engineer’s reference when troubleshooting that shows in the last 32 lines in the Door-opening Record.
(4). The Last-uploaded Door-opening Record will be saved in the Database. It is also the Last-uploaded Door-opening Record that be showed when clicking “Door-opening Record”. If wondering the new one, please operate as the following steps.

7.2. Steps for Door-opening Record Checking
Step1. Card issuing:
Click[image: image46.png]


 in the main interface(Click[image: image47.png]


(Put the Record Card (TEMIC LOCK is any kinds of TEMIC Card and Mifarel LOCK is S70 Card) on the Card Reader induction zone(Click[image: image48.png]/ lssue Card



.
Step2. Record downloading:

Download the Door-opening Record into Record Card (Mifarel LOCK) or Collection Device/Encoder/Collector (TEMIC LOCK).
If Mifarel LOCK, the card should be put over Door Lock’s Induction Zone 1cm and then the indicator LED will flash, still where it is for 20 seconds. When hearing one “beep”, the download procedure finishes successfully.

If TEMIC LOCK, Collector with four 7# batteries must have been prepared. Turn on Collector which green light will light on after two beeps. Swipe the Record Card over the Lock Sensor and put away, and pull the Collector near to Lock Sensor (as following picture) within 3 seconds, then the red LED of Collector will glitter regularly which stands for that the Collector is in the course of download that need about 30 Seconds. Turn off the Collector immediately after finishing collection.
[image: image49.png]



How to download the records to the collector
7.3. Record uploading:
If Mifarel LOCK, put the Record Card on the induction zone of Card Reader directly.

If TEMIC LOCK, connect the Collector to computer after finishing collection for downloading the records, after, turning it off. 
Follow the below steps to fetch the records:
Click[image: image50.png]7 Report



 (Select” Report of Open Door”(Click[image: image51.png]§ UplLoad



(Operate as the indication showed.
8. Receiption
Click[image: image52.png]


on the Main Window, there will be Reception Menu:
[image: image53.png]~ [Holiday Inn o] ‘ &b Room Status Setting ‘ Il Room Status Chart ‘ X Return]x]

10 & & & & A &
— 1001 1002 1003 i) 1006 1007 1008
B e & e e @] d
1011 1012 1013 1015 106, 1017 1018
ul @ | @ | @ | G @ @ 4§ 46
— __um 1102 1103 1108 1106 1107 1108 1100 1110
L=y L=y L=y L=y L=y L=y L=y
111 2 13 s 1116 amr g
12 2 2 2 2 2 2
— 1202 1203 1204 1208 1206 1207 1208 1200 1210
L=y L=y L=y L=y L=y L=y L=y L=y
1211 1212 1213 1214 1215 1216 1217 1218
13 L=y L=y L=y L=y L=y L=y L=y L=y
— __1m 1302 1303 1304 1308 1306 1307 1308 1300 1310
L=y L=y L=y L=y L=y L=y L=y L=y
1311 1312 1313 1314 1315 1316 1317 1318
14 L=y L=y L=y L=y L=y L=y L=y L=y
— __um 1402 1403 1404 1405 1406 1407 1408 1400 1410
L=y L=y L=y L=y L=y L=y L=y L=y
1411 1412 1413 1414 1415 1416 1417 1418
8 4 | & | s | » | & ]
Walkln  |Group Receiption| Reservation | Room Change Postpone | CK-Out with Card | Card Information |  Card Erase





8.1. Single Client Reception.
     Directly click the “Designated Room” or input the “Room Number” or look for “Room Search” for room selection. Please note that the system will default current building for not input the building Number.
[image: image54.png][F3]Room No. Walk In

1-1215 O Search

Standard

Vacant

I New Check-In & Cover Cards Before

Hour-rate Room

Days
1 B Detail >>
Expiry

2010- 1-26 o 12:00 = CIvIP

cards count: )

=@ [F5] Issue Card

@ CheckOutwithout Card

X Close




   
8.2. Group Reception.
[image: image55.png]Rooms Allocation

All Valid Rooms

11001
11002
11003
11004
1-1008
11010
11012
o ]
11111
11112
11113
11114
11115

1-1015
1-1009
11013

Rooms Selected G f

[F3]Room No.
1-1006 Group Checle-ln

Standard

Vacant

New Check-In & Cover Cards Before
Hour-rate Room

Days

1 El

Expiry

2010- 1-26 o) (1400 [o] MvIP

cards count: )

=& [F5] Issue Card

X Close

Team Leader

Name

Jordan

Sex Credential Type
Male < [Phone No.

Credential No.
00112345678

Price
123

Deposit
300

Memo
|

Check-ln

Check-Out




Step1: Input information of Tour Guide (the Name and IC Number at less).
Step2: Input Estimated Check-in Date or Estimated Check-out Time.

Step3: Appoint room.

Step4: Release card for each room.

      Group Check-out: Click “Group Reception” (Group Browse(Choose Tour Guide Name, Then Click[image: image56.png]@ oz





8.3. Reservation:
[image: image57.png]Rooms Allocation

Rooms Selected

[F3]Room No.
IMake Reservation

Reservation Check-n

2010- 125 [o] 1911 o

Days

1 B C Search
Expiry.

2010- 1-26 [o] 1200 [o] CIvIP

1-1006
1-1009
11010
1-1005
11012

cards count: )

=@ Make Reservation

@ cancel Reservation

X Close

Team Leader

Name

Lucy e
Sex Credential Type

Female  [2] [ID Card B

Credential No.
1234567890

Price
200

Deposit
3000/
Memo

Check-ln

Check-Out




             Step1: Input Information of Group Leader (the Name and IC Number at less).

         Step2: Input Estimated Check-in Time, Estimated Check-in Date or Estimated Check-out Time.

         Step3: Click “Room Search” to show all the available rooms in this period.
         Step4: Appoint room

         Step5: Click “New Reservation”. After the Reservation Setting success, Room Status will show as[image: image58.png]


.


Reservation Cancellation: Click “Reservation” (New Reservation(Select Name of Group Leader( clicks[image: image59.png]@ Ccancel Reservation




to cancel the rooms in the Sheet of Appointed Rooms. 

It also can be done if client just want to cancel some parts of rooms. Its operation as follows:
Remove do-not-want-to-canceled rooms from the sheet of “Designated Rooms” by double-clicking them. The rest of rooms in this sheet will be canceled.
Accommodation Cancellation: Click “Reservation” (Reserve Accommodation(Select the Name of Group Leader(Issue Card for each room.
8.4. Modify Room Status. 
There are Room Status Setup steps at the end of the interface.
[image: image60.png]&b Room Status Setting

FloorNo. [Room No. [Type Status Lock No. ~
10 100 Standard Vacant 01100199 4B vecantia
10 1002 Standard Vacant 01100299 =
10 1003 Standard Vacant 01100399 Hour Used (8]
10 1004 Standard Vacant 01100499
10 1005 Standard Hour Used 01100599 # Rented Remind [C]
10 1006 Standard Vacant 01100699

B ‘ Cleaning Room D]
10 1008 Standard Vacant 01100899
10 1009 Standard Vacant 01100999 @ Maintenance [E]
10 1010 Standard Vacant 01101099
10 1011 Standard Maintenance 01101199 @ Guest [F]
10 102 Standard Vacant 01101299
10 1013 Standard Vacant 01101399 m Guest Group [G]
10 1014 Standard Guest 01101499
10 1015 Standard Vacant 01101599 X Close
10 1016 Standard Vacant 1101689

Guide

Step 1: Select the guestroom that you want to set the status.
Do the one of the following method 1.Double click the guestroom 2.Use the Up/Down button to the guestroom, and then Enter 3.Press Shift +
wn button

means the selected guestrooms.

hoose the room status at the right-hand side, -> means Successful





Note: If, one room, for example, Check-out Time exceeds the computer’s current time, “Ask for Rent” Icon[image: image61.png]*
304




 will automatically display when refreshing Room Status. 
8.5. Room Status Diagram Checking:
[image: image62.png]il Room Status Chart

Holiday
Inn

I 181 Vacant

0 Reservation
B 1 Rented Remind|

190
180
170
160
150
140
130
120
110
100

1 Cleaning Room
1 Maintenance
1 Guest

I 12 Guest Group

80

50
a0
30
20

g oo
0

Vacant  Hour Used Rented Remind Maintenance  Guest  Guest Group




8.6. Card Information Inquiry
[image: image63.png]Guest Card [6411CCCF]

Date of Issuet 2010-1-25 19:15

Date of Expiy: 2010-1-26 12:00

Room o.: 1-1205

o 1
Holder: [Walkin]1-1205
Operator Code; Adinin
Status: New CheckIn & Cover Cards Befare,

Data In Card

Data In Database




8.7 Room Changing, Postpone, and Check-out With Card.
Put the Guest Card on Card Reader induction zone and click the relevant button and operate as the indication.
[image: image64.png]s » o

Room Change. Postpone | CK-Out with Card





8.8. Card Erase
Put the card on the Card Reader induction zone and click[image: image65.png]o

Card Erase



 to revoke the card information.
